|  | iRIC Software Changing River Science |  |
| --- | --- | --- |
|  | FaSTMECH Tutorial 2 |  |
|  | Green River, Dinosaur National Park, Colorado, USA |  |

Contents

Introduction	2

Getting Started	2

Pre-Processing	3

Importing Topography	3

Importing Measured Water Surface Elevations	5

Creating a Numerical Grid	5

Create grid	6

Mapping geographic data to the grid attributes.	8

Defining the Calculation Conditions and Running a Simulation	10

Post-processing	13

Map Visualizations	13

Displaying Scalar Results	13

Displaying Vector Results	15

Model Calibration and Refinement	16

Developing a Calibration Curve	16

Spatially Varying Drag Coefficient	19

Quasi-3D Model and 3D Graphics	23

Visualize Two- and Three-Dimensional Solution Without and With Streamline Curvature	23

Vertical Structure Continued	25

# Introduction

In this tutorial you will use the FaSTMECH solver in the iRIC application along with measured topography and water-surface elevations to model flow through a reach of the Green River in Dinosaur National Park, Utah, USA. This tutorial will demonstrate the following tasks necessary to complete a simulation:

- Getting Started

  - Introduction to navigating the iRIC interface

- Pre-processing

  - Importing geographic information

  - Creating a numerical grid

- Defining the Calculation Conditions and Running a Simulation

- Post-processing

  - Visualizing results

  - Graphing results

- Model Calibration and Model Refinement

  - Developing a calibration curve

  - Spatially variable roughness

# Getting Started

Launch iRIC by selecting iRIC from the Program Menu list or clicking on the iRIC icon on the desktop.

The iRIC Start Page (Figure 1A) opens and displays several options to start a project under the Start Simulation Project tab:

Create New Project—allows you to select the solver you wish to use from a list of solvers currently available in the application or to select the solver from a list of recently used solvers.

Open Project File—allows you to open an existing project using a browser window or to select from a list of recent projects.

Select the “Create New Project” button which opens the Select Solver dialog (Figure 1B). Highlight FaSTMECH and click the OK button.

| A | B |
| --- | --- |
| Shows the iRIC Start Page (A) and the Select Solver dialog (B). |  |

# Pre-Processing

## Importing Topography

The measured topography is the most important piece of information required to create a numerical model of the river reach of interest. Ideally the elevation data will extend well beyond the specific area of interest and be of sufficient spatial resolution for the application.

- Topography can be imported through the Menu Bar by selecting Import  Geographic Data  Elevation. In the Select File to Import dialog, navigate to the following folder:  iRIC Tutorials\FaSTMECH\Tutorials\Tutorial 2. iRIC can import several different file formats; for this tutorial please select “.tpo” in the Files of Type drop-down menu and select the following file: GR_Topo_Shifted.tpo. This will open a dialog that allows you to filter or reduce the number of points imported into iRIC. This can be useful if your data set is extremely large, but for our purposes leave the default setting at 1 and select OK to import the entire dataset.

- The Pre-processing Window now displays the topography data in the canvas and new data appears in the Object Browser under Geographic Data | Elevations | Points1 (Figure 2). In the Object Browser the topography can be made visible or not visible by checking or unchecking the box next to Elevation. Add a scalarbar from the Menu Bar by selecting Geographic Data  Set Up Scalarbar. Make sure Elevation is displayed in the drop-down menu and check the “Visible” box. Select OK (Figure 2).

Pre-processing Window display of the Elevation data.

- To adjust how the elevation points are displayed, highlight Elevation | Points1 in the Object Browser and right-click. This will bring up a dialog that allows you to rename the data in the Object Browser, Export the data, Delete the data, and adjust properties. Select “Property” and change the point size to 2 (Figure 3). Select OK when done.

Display Setting dialog to select graphic attributes for the Elevation data.

- Save (for example, named Tutorial 2) by selecting File  Save as File (*.ipro) from the Menu Bar.

- Import an image to place in the background of the data. Background images can be imported through the Menu Bar by selecting Import  Background Image. In the Open Image dialog select the Green River.jpg file. The image will be displayed in the canvas (Figure 4). To turn the image off, uncheck Background Images in the Object Browser.

Addition of the background imagery.

## Importing Measured Water Surface Elevations

- To verify the modeled water-surface elevation, you need to first import measured water-surface elevation data. Select Import  Measured Values from the Menu Bar.  Select the file “GR_wse.csv" in the File Open dialog. The data is added to the Pre-processing window Object Browser under Measured Values.

- You will see that importing the measured data adds a legend by default.  To turn the legend off in the Object Browser select and right-click on Measured Values | C:\(path)\GR_wse.csv | Scalar and in the resulting pop-up dialog select “Property”.  In the Scalar Setting dialog select the “Color Bar Setting” button, and then deselect the “Visible” attribute. Select OK to exit the Color Legend Setting dialog box and select OK again to exit the Scalar Setting dialog box.

## Creating a Numerical Grid

Most solvers require a computational or numerical grid to perform the calculations. iRIC contains a number of different methods or algorithms that can be used to generate the grid you need for your model. The FaSTMECH solver uses a structured curvilinear orthogonal grid. A curvilinear orthogonal grid defaults to a rectilinear grid when only two points define the centerline. This type of grid can be created in three basic steps: define the grid centerline, specify the width and density of points in the grid, and refine the curvature and location of the grid until a satisfactory result is achieved.

When creating a grid there are several things to keep in mind: 1) Keep the grid within the bounds of the data; 2) Define the upstream and downstream boundary of the grid beyond the area of computational interest to account for the effects of upstream and downstream boundary conditions; 3) Locate the upstream and downstream boundaries (the ends of the grid) in areas where flow is as uniform as possible. Avoid areas of recirculating flow or abrupt transitions in the bed or width; 4) Orient the upstream and downstream boundaries perpendicular to the main flow direction; 5) Select a grid width that minimizes the amount of the grid outside the active channel, therefore maximizing the number of nodes in the grid contributing to the solution; 6) Ensure that the grid does not overlap itself in the active channel. The application will give an error if the grid overlaps, which can cause issues with the solver later on.

### Create grid:

- To better view the data, click on the bottom right corner of the Pre-processing Window and drag to expand the window or select the maximize button along the top of the Pre-processing Window to expand the window.

- In the Menu Bar select Grid  Select Algorithm to Create Grid. This opens a dialog (Figure 5). Select “Create grid from polygonal line and width.” Click OK. Another dialog will open providing instructions on using this feature. Click OK.

Example of the Select Grid Creating Algorithm dialog.

- To draw the centerline, left-click in the desired locations starting at the upstream most point of interest and ending at the downstream (Figure 6). The upstream end of the channel in Figure 6 is at the bottom right and the channel centerline should be drawn in the direction from lower-right to upper-left. When finished, press “Enter” on the keyboard. Use Figure 6 and the blue water-surface points on the background image as a guide in selecting the boundary locations.

Location to draw centerline. Flow is from bottom to top so start at the lower end of the channel and select points ending near the top of the channel.

- A new dialog opens that allows you to specify the number of nodes in the streamwise direction, nI, the number of nodes in the cross-stream direction, nJ, and the width of the grid, W (Figure 7). Set the grid width equal to 360 meters and define the number of points in the streamwise and cross-stream dimension to give corresponding increments of about 10 meters (displayed by dI and dJ). The distance between nodes in the streamwise direction along the centerline and the cross-stream direction is constant everywhere on the grid. Use the “Apply” button on the dialog to dynamically change the view of the grid to find the desired spacing of nodes in the streamwise and cross-stream directions. Your values should be similar to those in Figure 7. Select OK when you are done. If you need to return to this menu, select Grid  Create Grid from the Menu Bar.

Grid Creation dialog that allows you to specify grid characteristics.

- A Confirmation message “Do you want to map geographic data to grid attributes now?” follows.  In this case we will decline by selecting “No”. We want to modify the location and curvature of the grid which is likely in this case to take many iterations. To disable the automatic mapping of geographic information from the Menu Bar select Grid  Attributes Mapping  Setting and in the resulting Grid Attribute Mapping Setting dialog, select “Manual” for the Execute mapping property. Select OK. Your grid should look similar to Figure 8.

Example grid for the Green River.

- To adjust the centerline to better fit the grid to the data or as in the case here, adjust the curvature of the grid to remove overlapping nodes, in the Object Browser select Grid Creating Condition. This is necessary to edit the grid. The centerline and points defining the centerline should be visible. When the mouse is placed over the centerline or over a centerline point, the mouse cursor changes to an open hand, and if the left mouse is clicked and dragged, it will change to a closed hand and move the centerline or centerline point. Experiment with adjusting centerline points to remove the overlapping grid nodes. If you make a mistake, Ctrl+z will undo and Ctrl+y will redo the previous action. Continue adjusting the centerline until there is no overlap of the grid.

- Adjust the grid so that the upstream and downstream boundaries are roughly perpendicular to the channel and the upstream and downstream extent of the grid encompasses the measured water-surface elevations.  Be careful at the upstream end because the measured water-surface elevation is right at the limit of the measured elevations. Extending the grid too far upstream will result in poor elevation values at the upstream end of the grid.

- Add or remove centerline points by selecting Grid  Grid Creating Condition on the Menu Bar and selecting the option you need. Be aware that new centerline points must be added along the current line; if you click too far away from the centerline no points will be added.

- Your result should look similar to Figure 9.

Edited grid location.

### Mapping geographic data to the grid attributes.

Once you are satisfied with your computational grid, you need to make sure that elevation is specified for each cell on the grid. There are two ways to do this. The first method uses a triangular irregular network (TIN) to interpolate the elevation data at each grid cell. The second method is based on a template that uses a nearest neighbor approach to interpolate. In a previous step we set the mapping algorithm to the template method.

- We need to set the width and length of the template. From the Menu Bar select Grid  Attributes Mapping  Setting. This opens the Grid Attribute Mapping Setting dialog.  We previously set the Execute attribute to “Manual” and the Mapping attribute to “Template mapping”. Select the “Detail” button near the bottom of the dialog (Figure 10).

Grid Attribute Mapping dialog

- Selecting “Detail” opens the Template Mapping Setting dialog. Set the Search region to “Manual” and set the other attributes to the same values as Figure 11 and select OK. The topography/bathymetry data set has a high point density, and we could simply use the option to map with the TIN, however, there is quite a bit of noise in the data so we choose here to slightly smooth out some of the noise by using the template with a size of 50m and 5m in the streamwise and cross-stream directions respectively. Select OK to exit the Grid Attribute Mapping Setting dialog.

Template Mapping Setting dialog

- To execute the mapping method, select Grid  Attributes Mapping  Execute from the Menu Bar. Check “Elevation” and select OK. Select OK when notified that mapping is complete. Node attributes will be added to the Object Browser under Grid () | Node Attributes. Expand the Node attributes and make sure that Elevation is selected.

- You will likely need to adjust the Color bar so you can view the results of mapping elevation to the grid. From the Object Browser select and right-click on Grid () | Node Attributes | Elevation and in the resulting pop-up menu select “Property”. In the Grid Node Attributes Display Setting dialog uncheck the box next to automatic. Set the max value to 449 and the min value to 444. Set the Contour setting attribute to “Contour Figure”. Select OK. Additionally, in the Object Browser, deselect Points, Grid Creating Condition, and Grid () | Grid Shape. Your results should look similar to Figure 12.

The resulting computational grid with the measured elevations mapped to the grid.

- Save the Project.

# Defining the Calculation Conditions and Running a Simulation

Once the grid is complete, the next step is to set up the required information, or calculation conditions, for the FaSTMECH solver.

- In the Menu Bar select Calculation Condition  Setting. This opens the Calculation Condition dialog. The left window of the dialog displays the primary parameter groups that can be set using the FaSTMECH solver. Values for various parameters are entered in the fields on the right side of the window.

- Enter the required parameters based on the values in Figure 13 and select “Save and Close” when complete.

| A | B |
| --- | --- |
| C | D |
| E | F |
| G | H |
| I | J |

FaSTMECH Calculation Conditions dialog. (A) Discharge, (B) Stage, (C) Roughness, (D) Lateral Eddy Viscosity, (E) Grid Extension, (F) Initial Conditions, (G) Wetting and Drying, (H) Solution Parameters, (I) Solution Relaxation, (J) 2D Solution Output.

- From the Menu Bar select Simulation  Run. The warning dialog will open to ask whether you wish to save the current project. Select “Yes”. After the project saves a Solver Console will open. This Console shows information about the simulation as the calculation is running (Figure 14). Note that the Solver Console displays the “mean error on discharge”, which is just the rms error between the discharge computed at each cross-section of the model compared to the desired discharge. Typically, this value should decrease as the simulation converges until it reaches a low value (typically values less than ~3 percent) and stabilizes. A dialog opens to notify you when the calculation is complete. Click OK to close the dialog.

- If the solution fails an error message will be displayed in the Solver Console and you must select  from the Main Toolbar and then in the Menu Bar select Calculation Conditions  Setting and make sure your settings are correct (see Figure 13). Also, make sure that your grid is inside the measured topography.

Example of the Solver Console.

# Post-processing

iRIC provides a complete suite of tools for visualizing 2D model results. Map visualizations of calculated flow characteristics are viewed in a 2D Post-processing Window. Graphs of calculated flow characteristics along different grid dimensions or through time can also be generated using the Graph Window tools. The Post-processing Windows and tools only become available when a simulation has been completed.

## Map Visualizations

Calculation results can be viewed by opening a new 2D Post-processing Window by selecting  on the Main Toolbar. The Post-processing Window is organized in a similar way as the Pre-processing Window with an Object Browser and canvas. The Object Browser in a Post-processing Window allows you to control the display of calculated flow characteristics such as depth, water-surface elevation, and velocity as well as to display arrows (vectors).

### Displaying Scalar Results

Scalar results show the magnitude of various flow characteristic through contour plots.

- If you haven’t already, open a 2D Post-Process Window by selecting  on the Main Toolbar.

- In the Object Browser select the check box next to iRICZone | Scalar (node) | WaterSurfaceElevation (Figure 15).

  - If the box is not available, you will have to add it into the Object Browser. This can be done by right-clicking iRICZone | Scalar (node) and selecting “Add”. Then in the Scalar Setting dialog, under the “Physical Value” drop-down menu, select “WaterSurfaceElevation”. Select OK to add.

Initial display of the simulated water-surface elevation.

- Change how the contours are displayed by right-clicking in the Object Browser FaSTMECH Grids | iRICZone | Scalar (node) | WaterSurfaceElevation (Figure 16). Experiment with attributes in the main Scalar setting dialog.

Scalar Setting dialog

- Change the spatial extent displayed by selecting “Region Setting” in the Scalar Setting dialog. Select “Active Region” to display only wet nodes (Figure 17A). The user can also select a specific area of interest to display using the “Custom Region” options. Select OK when done.

- Change the display of the legend or scalebar by selecting the “Color Bar Setting” button in the Scalar Setting dialog (Figure 17B). Select OK when done.

| A | B |
| --- | --- |

Additional display options available in the Scalar Setting dialog include the (A) Region Setting dialog and the (B) Color Legend Setting dialog (B).

- Experiment with these features to achieve a satisfactory display of the model solution (Figure 18).

Example display of the simulated water-surface elevation.

### Displaying Vector Results

Vector results such as the orientation and magnitude of velocity and shear stress can also be displayed using the 2D Post-processing Window.

- For now, in the Object Browser uncheck the box next to iRICZone | Scalar (node) to turn off the display of this type of data. Check the box next to iRICZone | Arrow and select iRIC Zone | Arrow | Velocity.

- The default values for the vectors often need adjustment. From the Menu Bar select Draw  Arrows to bring up the Arrow Setting dialog (Figure 19).  Adjust the values to those in Figure 19 and select OK when done. The result is shown in Figure 20.

The Vector Attributes dialog.

A plot of velocity in the modeled reach represented as vectors. Vectors are color coded and scaled according to their magnitude.

# Model Calibration and Refinement

## Developing a Calibration Curve

Model calibration is an important step in developing a flow model. Typically, simulated water-surface elevations are compared to measured water-surface elevation data collected at a known discharge. Comparing measured and simulated results helps to determine whether or not calculation conditions like roughness have been set appropriately. Other types of measured data can also be compared to the simulated values, such as velocity.

- In the Menu Bar select Calculation Result  Compare with measured values or select the  tool in the Main Toolbar. This opens a dialog that allows you to select which variables to compare.  Set the fields as shown in Figure 21, and select OK. A Verification Window opens that plots the measured and simulated water-surface elevations along the streamwise direction (Figure 22).

| Use the Verification Setting dialog to select data for comparison. If more than one measured data set has been imported there is an additional field to select which file to use. | Example verification plot showing the measured and simulated water-surface elevation. Note that the simulated water-surface elevation is too high compared to the measured data at the downstream end of the modeled reach. The Root Mean Squared Error (RMS) is 0.09 m. |
| --- | --- |

- To calibrate the model you could try adjusting the roughness value, Cd, to see if you can obtain a better match between the simulated and the measured values. Figure 23 shows the Verification plot that contains the RMS error for a range of simulated roughness values.  Looking at these results, it appears that a Cd of about 0.007 yields the lowest RMS error. However, none of the simulations are a good match at all locations.

| - Cd = 0.005 RMS = 0.102 m | B. Cd = 0.006 RMS = 0.066 m |
| --- | --- |
| C. Cd = 0.007 RMS = 0.065 m | D. Cd = 0.008 RMS = 0.090 m |
| E. Cd = 0.009 RMS = 0.127 m | F. Cd = 0.01 RMS = 0.166 m |
| Plots of the predicted and observed water-surface elevations for a range of drag coefficient values between 0.005 – 0.01 (A-F). The RMS error indicates that the best calibrated value for the drag coefficient is between 0.006 and 0.008. |  |

- Another way to look at these results is to plot a calibration curve which shows the roughness value on the x-axis and the RMS error on the y-axis (Figure 24). This curve indicates how sensitive the simulated water-surface elevation is to the roughness value. Additional simulations could further determine the drag coefficient that gives the lowest error between measured and predicted water-surface elevations.

Calibration curve showing the relationship between roughness and the RMS error for the simulated water-surface elevations.

## Spatially Varying Drag Coefficient

Examination of the simulated and measured water-surface elevations in Figure 23 indicates that using a constant drag coefficient for the entire reach yields unsatisfactory results. Generally, the predictions are good for either the upper or lower half of the reach but not the reach as a whole. This indicates that there is a change in roughness through the reach. This can be the result of many factors including a change in grain-size, different bedform geometries, and the presence of aquatic vegetation.

Maps of grain size distribution or vegetation can be used to adjust the roughness in specific regions. In this tutorial we simply vary the drag spatially by defining an area with a higher drag coefficient in the vicinity of the large island mid-way through the modeled reach. To spatially vary roughness, we’ll return to the Pre-processing Window. Note that Roughness is one of the Geographic data sets.

- The first step is to delineate polygons that spatially define regions of roughness.

  - In the Pre-processing Window turn on the background image and in the Object Browser right-click on Geographic Data | Roughness and in the resulting pop-up menu select Add  Polygon. Click OK when done reading the instructions and define a polygon that encloses the entire model Grid. When done, hit the “Enter” Key and enter a roughness value of 0.007 in the dialog. Select OK.

  - Create another polygon and this time define a polygon similar to the one in Figure 25. At the prompt enter a roughness value of 0.013. Select OK.

  - Make sure that both polygons are displayed in the Object Browser under Geographic Data | Roughness.

Example of the polygon mapping.

- The next step is to map or specify the roughness for each cell in the grid. The iRIC application knows when you have changed any values in the Object Browser and will make sure the new values are mapped before you run the simulation. However, in order to see how the polygons are mapped to the grid we’ll do it manually.

  - First highlight Geographic Data | Roughness in the Object Browser. Then on the Menu Bar select Grid  Attribute Mapping  Execute. Make sure Elevation and Roughness are checked. Select OK.

  - Make sure that the roughness values have been mapped to the grid by checking the box next to Roughness under Grid | Node Attributes in the Object Browser. You may need to expand those sections to see the full list. Make sure you get a result that looks like Figure 26.

  - You can edit the location of polygon nodes by selecting the polygon you want to edit in the Object Browser, and then much like the grid centerline you can move the mouse until it’s over a node to move the node or over a line between two nodes to move the whole polygon. You will also notice that there are tools in the Operation Toolbar to add or delete nodes in a polygon.

Final roughness attribute mapping

- Now update the calculation condition so that it uses a variable instead of a constant roughness value.

  - From the Menu Bar select Calculation Condition  Setting. In the Calculation Condition dialog select roughness and change the settings to look like Figure 27. Select the “Save and Close” button.

Calculation condition, Roughness dialog

- Now rerun the simulation by clicking the run button or by selecting Simulation   Run from the Menu Bar.

- When the simulation is complete, look at the Verification between the calculated WaterSurfaceElevation results and the measured values of the WaterSurfaceElevation (Figure 28).

Comparison of the measured and simulated water-surface elevation using a spatially variable drag coefficient. RMS error is 0.055 m.

# Quasi-3D Model and 3D Graphics

In this section, the simple vertical structure of shear flows and the generation of secondary flows using the quasi-3D sub-model available in FaSTMECH is briefly explored.  A more in-depth discussion of this topic is available in Tutorial 3. In addition, an introduction to the 3D graphics capabilities of iRIC is provided here. The model previously developed is used and the quasi-3d model and 3d solution is simply turned on. A nice feature of this function is that the quasi-3d solution is computed upon completion of the 2d solution, such that this additional step requires a minor amount of additional computation time.  The quasi-3d model offers the option of using the channel curvature or channel curvature + streamline curvature for calculating secondary flows. We’ll use both options in the following sections.  First, secondary flow is expressed as the helix strength, which is defined as the angular difference between the near-bed flow and the surface flow and is a simple measure of the presence and strength of secondary flows is explored.  Second, vertical sections of velocity vectors are plotted to illustrate the rotation of vectors in the vertical due to secondary flow.  Finally, cross-sections are examined through the reach to visualize the vertical structure of the flow.

### Visualize Two- and Three-Dimensional Solution Without and With Streamline Curvature

- Return to the Pre-processing Window by selecting View  Pre-processing Window from the Menu Bar.

- Select Calculation Condition  Setting in the Menu Bar and edit the following tabs in the Calculation Condition dialog as in Figures 29 and 30.  In this first case only channel curvature is used.  3D output including the calculation of the helix strength and the 3D scalar and vector quantities are added through the 3D Solution Output tab. Select “Save and Close” when done.

| Quasi-3D Solution tab of the Calculation Conditions. | 3D Solution Output tab of the Calculation Conditions |
| --- | --- |

- Re-run the model to obtain the new solution.  Notice that in the toolbar the 3D () option is now available.

- Before exploring the 3D graphing options, look at the Helix Strength 2D solution result as in Figure 31 below.

  - Set the min and max Helix strength to -25 and 25 respectively.  Also, set the Display Property setting to Contour.

Re-run the model with the “Use Streamline curvature” option set in the Calculation Conditions and the Minimum Streamline Curvature set to 10m.  The resulting helix strength solution is plotted in Figure 32.

- The helix strength and streamlines are plotted in Figure 33.  Note the correlation between curvature of the streamlines and the helix strength.

Helix strength calculated using only the grid curvature

Helix strength calculated using streamline curvature

Helix strength and streamlines originating from the upstream boundary.  Notice the correlation between curvature in the streamlines and the helix strength.  The boxes are the locations of the following figures: A) Figure 34, B) Figure 35) and C) Figure 36.

### Vertical Structure Continued

- To create an image as shown in Figure 34:

- In the 3D Post Processing window right-click FaSTMECH Grids | iRICZone | Arrow in the Object Browser and select the Property pop-up menu. Set up the Arrow Setting dialog as shown in Figure 34 (A and B).  Where each Face added (Face001, Face002…, Face011) incrementally increases the value of the K index (from 1 – 11).  Also, set up the Length, Sampling, and Color fields as shown.

- By zooming into the downstream end of the main (river-left) channel around the island, where the flow has some of the highest curvature, you should be able to reproduce the graphics as shown in Figure 34 (C). The velocity from every vertical node is shown illustrating the secondary flow where the surface velocities are oriented towards the outside of the bend and the near-bed velocities are oriented towards the inside of the bend.

| A | B |
| --- | --- |
| C |  |

The Arrow Setting dialog.  Note that in (A) the K index is set to 1 and in (B) the K index is set to 11. The 3-dimensional vector field where the velocities at the surface are highest and the near bed velocities are lowest.  In (C), notice how the orientation of the velocities changes as the streamline curvature changes.

- To Create an image as shown in Figure 35:

- In the 3D Post Processing window right-click FaSTMECH Grids | iRICZone | Contours in the Object Browser and select the Add or Property pop-up menu. Set up the Contour Setting dialog as shown in Figure 35 (A & B). Note that the NVelocity is the cross-stream velocity relative to the grid coordinate system.  When the curvature of the coordinate system (Grid) follows the channel curvature nicely it’s easy to see the secondary flow in the plots below (see Figure 36).  Seven faces are added incrementally with the value of the I-index equal to 200, 210, 220, 230, 240, 250, 260.

- Additionally, an iso-surface of the velocity magnitude (0.01 m/s) is added to provide a sense of the channel bottom (Figure 35C).

- To add some vertical exaggeration so that it’s easier to see the cross-sections select View  Z Direction Scale from the Menu Bar and enter “5” in the Z-direction scale. The easiest way to orient the channel is to first select the  button and then using the Ctrl-key and right mouse button – drag the mouse from top to bottom.  In this case flow is from top to bottom.  Convince yourself of this with the knowledge that the coordinate system is right-handed.

- Figure 36 is created by adding additional contour sections with the I-index values equal to 125, 130, 135, and 140.  The max and min NVelocity values are 0.1 to -0.1.

| A | B |
| --- | --- |
| C |  |
| D |  |

The cross-stream velocity is shown for several cross-sections through the reach.  NVelocity is the magnitude of the velocity oriented perpendicular to the streamwise direction.  When the grid follows the channel curvature well, NVelocity provides a good representation of the secondary flow.  Here the grid does not follow the curvature of each channel around the island, so we only capture some of the secondary flow with these plots.

Vertical Profiles at cross-sections corresponding to I-locations of 125, 130, 135, and 140.

- For a more in-depth discussion of secondary flows and its effect on sediment transport, please see Tutorials 3 and 5, respectively.